ORCP

Open Robot Control Protocol

Specification

Version 1.0

March 2026

A simple, safe, transport-agnostic
protocol for robot control.

This specification is released under the MIT License.

You are free to implement, modify, and distribute implementations

of this protocol without restriction.

# Table of Contents

Table of Contents	2

1. Introduction	4

1.1 Purpose	4

1.2 Scope	4

1.3 Design Principles	4

1.4 Terminology	5

1.5 Normative Language	5

2. ASCII Message Format	6

2.1 Overview	6

2.2 Commands (Host → Controller)	6

2.3 Success Response (Controller → Host)	6

2.4 Error Response (Controller → Host)	6

2.5 Push Messages (Controller → Host, Unsolicited)	6

2.6 Error Codes	6

3. Transport Layer	8

3.1 General Requirements	8

3.2 USB CDC (Recommended Default)	8

3.3 TCP Socket (WiFi / Ethernet)	8

3.4 UART	8

3.5 CAN Bus	8

4. Command Reference	9

CMD_VEL	9

DEFAULTS	9

ENABLE	10

GET	10

HB	11

INFO	11

LOAD	11

PING	12

PRESET	12

SAVE	13

SET	13

STATUS	14

STOP	15

STREAM	15

WHEEL	16

5. Safety System	17

5.1 Operating Presets	17

5.2 Fault Hierarchy	17

5.3 Emergency Stop	17

5.4 Command Timeout	17

5.5 Heartbeat	18

5.6 Acceleration Limiting	18

6. Push Messages	19

6.1 Streaming Telemetry	19

6.2 Warnings	19

6.3 Faults	19

7. Standard Configuration Parameters	20

7.1 Kinematics	20

7.2 PID Controller	20

7.3 Battery Monitoring	20

7.4 Safety Timing	20

8. Kinematic Model	20

9. Conformance Levels	22

9.1 Level 1 — Basic	22

9.2 Level 2 — Standard	22

9.3 Level 3 — Extended	22

10. Units	23

Annex A: CAN Binary Encoding (Normative)	24

A.1 Addressing	24

A.2 Value Encoding	24

A.3 Command Frames (Host → Controller)	24

A.4 Response Frames (Controller → Host)	24

A.5 Parameter IDs	25

Annex B: Extending ORCP (Informative)	26

B.1 Vendor-Specific Commands	26

B.2 Additional Telemetry Fields	26

B.3 Additional Kinematic Models	26

B.4 Multi-Robot Coordination	26

Revision History	27

License	27

# 1. Introduction

## 1.1 Purpose

The Open Robot Control Protocol (ORCP) defines a standard interface for controlling and monitoring robots. It provides a common language between a high-level controller (a laptop, single-board computer, PLC, or fleet management system) and a low-level device controller (a microcontroller managing actuators, sensors, and safety systems). ORCP v1.0 focuses on mobile platform motion control, with future versions extending to sensor integration, manipulator control, and multi-robot coordination.

ORCP is designed to be simple enough to type in a terminal, structured enough for real-time control, safe enough for deployment in educational and commercial environments, and extensible enough for research and industrial applications.

## 1.2 Scope

ORCP covers the following functional areas:

Motion control: Velocity commands for differential-drive and skid-steer mobile bases, using both unicycle (linear/angular) and direct wheel velocity models.

Safety management: Operating mode presets, enable gates, heartbeat monitoring, emergency stop integration, and configurable fault handling.

Telemetry: Real-time streaming of velocities, duty cycles, battery status, and sensor data.

Configuration: Runtime parameter management with persistent storage for tuning, calibration, and deployment.

System information: Device identification, capability reporting, and diagnostics.

ORCP does not define motor driver hardware, mechanical design, kinematics beyond differential drive, path planning, or SLAM algorithms. It is a device-level protocol, not a middleware or application framework.

## 1.3 Design Principles

Human-readable: The primary encoding is ASCII text. Commands can be typed and responses read in any serial terminal. No special tools are required to operate an ORCP device.

Transport-agnostic: ORCP defines message semantics independently of the physical transport. Normative encodings are provided for ASCII (over USB CDC, UART, or TCP) and binary (over CAN bus). Other transports may be used provided the message semantics are preserved.

Named parameters: All parameters use key=value syntax. Parameters are order-independent and self-documenting. Clients MUST ignore unrecognised parameters in responses.

Forward-compatible: Implementations MAY add new parameters to existing responses and new commands to the protocol without breaking conformant clients, provided existing semantics are preserved.

Fail-safe: Unrecognised commands MUST return an error. The safety system MUST enforce timeouts. The device MUST stop if communication is lost.

## 1.4 Terminology

This specification uses the following terms:

Controller: The device implementing ORCP — typically a microcontroller board driving motors and reading sensors. Also called the 'device' or 'server'.

Host: The system sending commands to the controller — a laptop, SBC, PLC, or any software client. Also called the 'client'.

Base: The mobile robot platform containing the controller, motors, wheels, battery, and safety systems.

Transport: The physical and data-link layer carrying ORCP messages (USB CDC, TCP socket, UART, CAN bus, etc.).

## 1.5 Normative Language

The key words MUST, MUST NOT, SHALL, SHALL NOT, SHOULD, SHOULD NOT, RECOMMENDED, MAY, and OPTIONAL in this document are to be interpreted as described in RFC 2119.

# 2. ASCII Message Format

## 2.1 Overview

ORCP uses a line-oriented ASCII message format. Each message is a single line of UTF-8 text terminated by a newline character. The protocol follows a request-response model with support for unsolicited push messages from the controller.

## 2.2 Commands (Host → Controller)

COMMAND [param=value ...] <newline>

COMMAND: An uppercase ASCII keyword identifying the operation.

Parameters: Zero or more space-separated key=value pairs. Keys are lowercase. Values may be integers, floats, or strings depending on the parameter.

Terminator: LF (0x0A) or CR+LF (0x0D 0x0A). Implementations MUST accept both.

Maximum line length: 256 bytes including terminator.

## 2.3 Success Response (Controller → Host)

OK COMMAND [field=value ...] <newline>

The response echoes the command keyword and includes zero or more result fields. Clients MUST ignore unrecognised fields to ensure forward compatibility.

## 2.4 Error Response (Controller → Host)

ERR code=<ERROR_CODE> msg="<description>" <newline>

Returned when a command cannot be executed. The error code is a machine-readable keyword. The message is a human-readable description.

## 2.5 Push Messages (Controller → Host, Unsolicited)

! TYPE [field=value ...] <newline>

Push messages are sent asynchronously by the controller without a preceding command. They are prefixed with '!' followed by a space and a type keyword. Hosts MUST be prepared to receive push messages at any time, including between sending a command and receiving its response.

## 2.6 Error Codes

| Code | Description |
| --- | --- |
| BAD_CMD | Unrecognised command keyword |
| BAD_ARG | Missing or malformed parameter |
| BAD_VAL | Parameter value out of valid range |
| NOT_ENABLED | Motion rejected: safety enable gate not set |
| ESTOP | Motion rejected: emergency stop active |
| LOWBATT | Motion rejected: battery critically low |
| HEARTBEAT | Motion rejected: heartbeat timeout |
| TIMEOUT | Motion rejected: command timeout |
| BUSY | Command cannot execute in current state |
| FLASH_ERR | Persistent storage read/write error |

Implementations MAY define additional error codes. Clients SHOULD handle unknown error codes gracefully by displaying the code and message to the user.

# 3. Transport Layer

## 3.1 General Requirements

ORCP is transport-agnostic. Any reliable byte-stream or message-based transport may be used. The ASCII encoding (Section 2) is normative for stream-based transports. The CAN binary encoding (Annex A) is normative for CAN bus. Implementations MUST specify which transports they support.

## 3.2 USB CDC (Recommended Default)

The controller presents as a USB CDC ACM device (virtual COM port). No special drivers are required on modern operating systems. The baud rate setting (typically 115200) is required by the USB CDC specification but has no effect on actual throughput, as USB CDC operates at full USB speed.

## 3.3 TCP Socket (WiFi / Ethernet)

The controller listens on a TCP port (default 3333). The host connects as a TCP client. Multiple simultaneous clients SHOULD be supported (RECOMMENDED minimum: 4). Each client maintains an independent session. Push messages (streaming telemetry) SHOULD be sent to all connected clients.

Service discovery via mDNS is RECOMMENDED. The default service type is _orcp._tcp and the default hostname is the device's configured name.

## 3.4 UART

For direct microcontroller-to-microcontroller communication. Settings: 115200 baud, 8 data bits, no parity, 1 stop bit (8N1). Hardware flow control is OPTIONAL.

## 3.5 CAN Bus

CAN 2.0B at up to 1 Mbit/s. Messages use the binary encoding defined in Annex A. Each device on the bus is identified by a configurable base ID. Multiple devices MAY share a single CAN bus using different base IDs.

# 4. Command Reference

This section defines all ORCP commands. Commands are listed alphabetically. An implementation that supports a given conformance level (Section 9) MUST implement all commands marked as required for that level.

### CMD_VEL

Category: Motion

Set the robot's linear and angular velocity using the unicycle kinematic model. The controller converts these to individual wheel velocities using its calibrated kinematics (wheel radius, track width). This is the primary motion command for most applications.

Syntax:

CMD_VEL v=<float> w=<float>

| Parameter | Type | Required | Description |
| --- | --- | --- | --- |
| v | float | Yes | Linear velocity in m/s. Positive = forward. |
| w | float | Yes | Angular velocity in rad/s. Positive = counter-clockwise. |

Response: OK CMD_VEL v=<f> w=<f> tl=<f> tr=<f>

Example:

>>> CMD_VEL v=0.200 w=0.500

<<< OK CMD_VEL v=0.200 w=0.500 tl=2.271 tr=5.893

The controller MAY implement acceleration limiting. If present, the actual PID setpoint ramps towards the commanded target. The STOP command MUST bypass any acceleration limiter for instant braking.

### DEFAULTS

Category: Configuration

Reset all configuration parameters to factory default values. Does not automatically persist — the host MUST send SAVE to make the reset permanent.

Syntax:

DEFAULTS

Response: OK DEFAULTS

Example:

>>> DEFAULTS

<<< OK DEFAULTS

Implementations MUST define sensible defaults for all parameters.

### ENABLE

Category: Safety

Set or clear the motor safety enable gate. In restricted operating modes, motion commands are rejected until the enable gate is set. In permissive modes, the gate may be set automatically at power-on.

Syntax:

ENABLE <ON|OFF>

| Parameter | Type | Required | Description |
| --- | --- | --- | --- |
| ON | keyword | Yes | Enable the motor safety gate. Clears recoverable faults. |
| OFF | keyword | Yes | Disable the motor safety gate. Motors stop immediately. |

Response: OK ENABLE state=<ON|OFF>

Example:

>>> ENABLE ON

<<< OK ENABLE state=ON

ENABLE ON MUST be rejected with ERR code=ESTOP if the emergency stop is active.

### GET

Category: Configuration

Read the current value of a configuration parameter. See Section 7 for the standard parameter set.

Syntax:

GET <parameter_name>

| Parameter | Type | Required | Description |
| --- | --- | --- | --- |
| parameter_name | string | Yes | Name of the parameter to read. |

Response: OK GET <name>=<value>

Example:

>>> GET kp

<<< OK GET kp=0.050

Implementations SHOULD support GET ALL to return all parameters.

### HB

Category: Safety

Send a heartbeat to the safety system. In modes that require heartbeat monitoring, the host MUST send HB commands periodically. If the heartbeat timeout expires, the controller MUST stop the motors and raise a HEARTBEAT fault.

Syntax:

HB

Response: OK HB

Example:

>>> HB

<<< OK HB

The heartbeat also resets the command timeout timer. A typical heartbeat interval is 100 ms.

### INFO

Category: System

Return device identification and protocol version.

Syntax:

INFO

Response: OK INFO fw=<version> hw=<hardware> proto=ORCP/<version> [vendor=<name>] [model=<name>]

Example:

>>> INFO

<<< OK INFO fw=1.0.0 hw=STM32F103CB proto=ORCP/1.0 vendor=Acme model=BaseBot

The proto field MUST include the ORCP version. The vendor and model fields are OPTIONAL.

### LOAD

Category: Configuration

Load configuration parameters from persistent storage, replacing current in-memory values.

Syntax:

LOAD

Response: OK LOAD

Example:

>>> LOAD

<<< OK LOAD

If no saved configuration exists, LOAD SHOULD apply factory defaults.

### PING

Category: System

Test connectivity. The simplest possible command. If you get a response, the link is working.

Syntax:

PING

Response: OK PONG

Example:

>>> PING

<<< OK PONG

Implementations MUST respond to PING regardless of safety state, faults, or operating mode.

### PRESET

Category: Safety

Switch the safety system between operating modes. Presets configure the duty limit, command timeout, heartbeat requirement, and enable gate behaviour as a single atomic operation. Motors MUST be stopped when switching presets.

Syntax:

PRESET <preset_name>

| Parameter | Type | Required | Description |
| --- | --- | --- | --- |
| preset_name | string | Yes | Name of the safety preset to activate. |

Response: OK PRESET name=<name> timeout_ms=<int> enable_required=<0|1> hb_required=<0|1> duty_limit=<float>

Example:

>>> PRESET NORMAL

<<< OK PRESET name=NORMAL timeout_ms=250 enable_required=1 hb_required=1 duty_limit=1.000

Implementations MUST define at least two presets: a restricted mode (low power, auto-enabled) and a full mode (full power, requires explicit enable and heartbeat). The standard preset names are SLOW and NORMAL.

### SAVE

Category: Configuration

Save current configuration parameters to persistent storage. Parameters persist across power cycles.

Syntax:

SAVE

Response: OK SAVE

Example:

>>> SAVE

<<< OK SAVE

Implementations SHOULD use integrity checking (e.g. CRC32) on stored data. Flash write endurance is limited; hosts SHOULD NOT call SAVE in a loop.

### SET

Category: Configuration

Change a configuration parameter in memory. The change takes effect immediately but does not persist across power cycles unless followed by SAVE.

Syntax:

SET <parameter_name>=<value>

| Parameter | Type | Required | Description |
| --- | --- | --- | --- |
| parameter_name | string | Yes | Name of the parameter to change. |
| value | varies | Yes | New value. Type depends on the parameter. |

Response: OK SET <name>=<value>

Example:

>>> SET kp=0.080

<<< OK SET kp=0.080

Implementations MUST validate the value and return ERR code=BAD_VAL if out of range.

### STATUS

Category: System

Return a complete snapshot of the device state: safety mode, control mode, velocities, duty cycles, battery status, and active faults.

Syntax:

STATUS

Response: OK STATUS preset=<P> mode=<M> en=<0|1> fault=<F> estop=<0|1> tl=<f> tr=<f> vl=<f> vr=<f> dl=<f> dr=<f> lim=<f> vbat=<f> battery=<B>

Example:

>>> STATUS

<<< OK STATUS preset=SLOW mode=IDLE en=1 fault=OK estop=0 tl=0.000 tr=0.000 vl=0.000 vr=0.000 dl=0.000 dr=0.000 lim=0.300 vbat=12.45 battery=HIGH

Implementations MAY include additional fields. Clients MUST ignore unrecognised fields.

Standard STATUS response fields:

| Field | Type | Description |
| --- | --- | --- |
| preset | string | Active safety preset name |
| mode | string | Control mode: IDLE, VELOCITY, or OPEN_LOOP |
| en | 0¦1 | Safety enable gate state |
| fault | string | Active fault code or OK |
| estop | 0¦1 | Emergency stop state (1 = active) |
| tl, tr | float | Target velocity left/right (rad/s) |
| vl, vr | float | Measured velocity left/right (rad/s) |
| dl, dr | float | Duty cycle left/right (−1.0 to +1.0) |
| lim | float | Active duty limit (0.0 to 1.0) |
| vbat | float | Battery voltage (V) |
| battery | string | Battery level (percentage or band label) |

### STOP

Category: Motion

Immediately stop all motors using active braking. MUST bypass any acceleration limiter for instant response. MUST be accepted regardless of safety state — STOP never fails.

Syntax:

STOP

Response: OK STOP mode=BRAKE

Example:

>>> STOP

<<< OK STOP mode=BRAKE

STOP does not disable the safety enable gate. After STOP, the host can immediately send a new motion command.

### STREAM

Category: Telemetry

Enable or disable continuous telemetry streaming. When enabled, the controller pushes telemetry at the specified rate.

Syntax:

STREAM <ON|OFF> [rate]

| Parameter | Type | Required | Description |
| --- | --- | --- | --- |
| ON | keyword | Yes | Enable streaming. |
| OFF | keyword | Yes | Disable streaming. |
| rate | int (1–50) | No | Stream rate in Hz. Default: 10. |

Response: OK STREAM state=<ON|OFF> rate=<int>

Example:

>>> STREAM ON 20

<<< OK STREAM state=ON rate=20

Stream data is sent as push messages with prefix '! STREAM'. The controller SHOULD maintain timing accuracy within ±10% of the requested rate.

Standard streaming telemetry format:

! STREAM tl=5.000 tr=5.000 vl=4.987 vr=5.012 dl=0.142 dr=0.143 vbat=12.44 battery=HIGH

Implementations MAY include additional fields (e.g. current monitoring, IMU data). Clients MUST ignore unrecognised fields.

### WHEEL

Category: Motion

Set individual wheel side velocities directly in rad/s of the output shaft. This bypasses the unicycle kinematic model and gives direct control over each wheel side's PID controller. For skid-steer and 4WD platforms, 'left' and 'right' refer to the entire side, not individual wheels.

Syntax:

WHEEL l=<float> r=<float>

| Parameter | Type | Required | Description |
| --- | --- | --- | --- |
| l | float | Yes | Left side velocity in rad/s. Positive = forward. |
| r | float | Yes | Right side velocity in rad/s. Positive = forward. |

Response: OK WHEEL l=<f> r=<f>

Example:

>>> WHEEL l=5.0 r=-5.0

<<< OK WHEEL l=5.000 r=-5.000

Acceleration limiting, if implemented, applies to WHEEL commands the same as CMD_VEL.

# 5. Safety System

ORCP defines a mandatory safety system that MUST be implemented by all conformant controllers. The safety system prevents uncontrolled motion and ensures the robot can be stopped at any time.

## 5.1 Operating Presets

Presets group safety parameters into named configurations that can be switched atomically. Implementations MUST define at least two presets:

| Feature | Restricted (e.g. SLOW) | Full (e.g. NORMAL) |
| --- | --- | --- |
| Duty limit | Reduced (≤30%) | Full (100%) |
| Command timeout | Lenient (≥1000 ms) | Tight (≤250 ms) |
| Auto-enabled | Yes | No (requires ENABLE ON) |
| Heartbeat required | No | Yes |
| Intended use | Manual testing, beginners | Autonomous operation, production |

The controller MUST power on in the restricted preset. This ensures safe default behaviour.

## 5.2 Fault Hierarchy

The safety system MUST check for faults on every control tick and report the highest-priority active fault. Faults MUST be checked in the following priority order:

| Priority | Fault | Cause | Recovery |
| --- | --- | --- | --- |
| 1 (highest) | ESTOP | Emergency stop hardware signal active | Release, then ENABLE ON |
| 2 | LOWBATT | Battery below critical threshold | Recharge, then ENABLE ON |
| 3 | NOT_ENABLED | Enable gate not set (full mode) | ENABLE ON |
| 4 | HEARTBEAT | Heartbeat timeout (full mode) | HB, then ENABLE ON |
| 5 (lowest) | TIMEOUT | No motion command within timeout | Send any motion command |

## 5.3 Emergency Stop

ORCP controllers SHOULD support a hardware emergency stop input. When active, the controller MUST immediately stop all motors and reject all motion commands with ERR code=ESTOP. The ENABLE ON command MUST also be rejected while the emergency stop is active.

After the emergency stop is released, the fault MUST persist until the host explicitly sends ENABLE ON. Motors MUST NOT restart automatically after e-stop release.

## 5.4 Command Timeout

If no motion command (CMD_VEL, WHEEL) or heartbeat (HB) is received within the configured timeout period, the controller MUST stop the motors. This protects against host software crashes, network disconnections, or interrupted programs.

## 5.5 Heartbeat

In the full operating preset, the host MUST periodically send HB commands to prove it is alive and in control. The recommended interval is 100 ms with a default timeout of 500 ms. If the timeout expires, the controller MUST stop the motors and raise a HEARTBEAT fault.

## 5.6 Acceleration Limiting

Implementations SHOULD include a configurable acceleration limiter that ramps the PID setpoint towards the commanded target at a maximum rate. This prevents wheel spin, reduces mechanical stress, and improves controllability. The STOP command MUST always bypass the acceleration limiter for instant braking.

# 6. Push Messages

Push messages are unsolicited messages from the controller to the host. They are prefixed with '!' and MUST be handled asynchronously. Push messages MAY arrive at any time, including between a command and its response.

## 6.1 Streaming Telemetry

! STREAM [field=value ...]

Sent at the configured rate when STREAM ON is active.

## 6.2 Warnings

! WARN <type> [field=value ...]

Proactive notification of a degrading condition (e.g. low battery). The robot is still operational.

## 6.3 Faults

! FAULT <fault_code>

Notification that a safety fault has been triggered and motors have been stopped.

# 7. Standard Configuration Parameters

This section defines the standard parameter set. Implementations MUST support all parameters marked Required for their conformance level. Implementations MAY define additional vendor-specific parameters.

## 7.1 Kinematics

| Parameter | Type | Unit | Required | Description |
| --- | --- | --- | --- | --- |
| counts_per_rev | int | counts | Yes | Encoder counts per full output shaft revolution |
| wheel_diameter | float | mm | Yes | Wheel diameter |
| track_width | float | mm | Yes | Distance between wheel centres (or side centres for skid-steer) |
| max_accel | float | rad/s² | No | Maximum wheel acceleration rate (0 = disabled) |

## 7.2 PID Controller

| Parameter | Type | Unit | Required | Description |
| --- | --- | --- | --- | --- |
| kp | float | — | Yes | Proportional gain |
| ki | float | — | Yes | Integral gain |
| kd | float | — | Yes | Derivative gain |

## 7.3 Battery Monitoring

| Parameter | Type | Unit | Required | Description |
| --- | --- | --- | --- | --- |
| batt_full | float | V | No | Full charge voltage (for level estimation) |
| batt_low | float | V | No | Low warning threshold |
| batt_critical | float | V | Yes* | Critical threshold (motors disabled). *Required if battery monitoring is implemented. |

## 7.4 Safety Timing

| Parameter | Type | Unit | Required | Description |
| --- | --- | --- | --- | --- |
| slow_timeout | int | ms | Yes | Command timeout in restricted mode |
| normal_timeout | int | ms | Yes | Command timeout in full mode |
| hb_timeout | int | ms | Yes | Heartbeat timeout in full mode |
| slow_duty_limit | float | 0–1 | Yes | Duty limit in restricted mode |
| normal_duty_limit | float | 0–1 | Yes | Duty limit in full mode |

# 8. Kinematic Model

ORCP assumes a differential-drive or skid-steer kinematic model. The conversion from linear/angular velocity (CMD_VEL) to wheel velocities is:

wheel_left  = (v - w * track_width / 2) / wheel_radius

wheel_right = (v + w * track_width / 2) / wheel_radius

Where v is linear velocity (m/s), w is angular velocity (rad/s), track_width is the distance between wheel centres (m), and wheel_radius is the wheel radius (m). The result is in rad/s of the output shaft.

For skid-steer (4WD) platforms, the same model applies. Both motors on the left side receive wheel_left, and both motors on the right side receive wheel_right. The track_width parameter represents the effective track width, which may differ from the physical wheel spacing due to scrub effects.

Implementations MAY support other kinematic models (Ackermann, omnidirectional) as extensions. Such extensions SHOULD define new commands rather than overloading CMD_VEL with incompatible semantics.

# 9. Conformance Levels

ORCP defines three conformance levels. An implementation MUST state which level it conforms to.

## 9.1 Level 1 — Basic

Minimum viable ORCP implementation. Suitable for hobby projects and simple platforms.

Required commands: PING, CMD_VEL, WHEEL, STOP, STATUS, ENABLE, PRESET

Required presets: At least two (restricted and full)

Required safety: Command timeout, enable gate

Configuration: Not required (parameters may be compile-time only)

## 9.2 Level 2 — Standard

Full ORCP implementation. Suitable for education and research.

Required commands: All Level 1 commands plus INFO, HB, STREAM, GET, SET, SAVE, LOAD, DEFAULTS

Required safety: All Level 1 safety plus heartbeat monitoring, e-stop support

Configuration: Full GET/SET with persistent storage

Telemetry: STREAM with configurable rate

## 9.3 Level 3 — Extended

ORCP with additional capabilities. Suitable for commercial and multi-robot applications.

Required: All Level 2 features

CAN binary encoding: Support for Annex A CAN transport

Multi-robot: Configurable node ID for bus-based transports

Diagnostics: Per-motor current reporting, fault logging

# 10. Units

ORCP uses SI units throughout. All implementations MUST use the following units:

| Quantity | Unit | Notes |
| --- | --- | --- |
| Linear velocity | m/s | CMD_VEL v parameter |
| Angular velocity | rad/s | CMD_VEL w and WHEEL l/r parameters |
| Wheel acceleration | rad/s² | max_accel parameter |
| Distance | mm | wheel_diameter, track_width (mm for human readability) |
| Duty cycle | −1.0 to +1.0 | Fraction of maximum PWM |
| Voltage | V | Battery voltage |
| Current | mA | Current measurements |
| Time | ms | Timeout parameters |
| Frequency | Hz | Stream rate |
| Encoder resolution | counts | Per full output shaft revolution |

# Annex A: CAN Binary Encoding (Normative)

This annex defines the binary encoding of ORCP messages for CAN 2.0B transport. Each ORCP command and response maps to one or more CAN frames. Values are encoded as fixed-point integers for efficiency.

## A.1 Addressing

Each ORCP device on a CAN bus is identified by a configurable base ID (default 0x100). Command and response message IDs are offsets from this base. This allows up to 16 devices on a single bus with base IDs spaced 0x20 apart (0x100, 0x120, 0x140, ...).

## A.2 Value Encoding

Velocities (m/s, rad/s): Multiplied by 1000, stored as int16. Range ±32.767 with 0.001 resolution.

Voltages (V): Multiplied by 1000, stored as uint16. Range 0–65.535 V.

Currents (mA): Stored as uint16.

Duty cycle: Multiplied by 1000, stored as int16.

Byte order: Little-endian (LSB first), consistent with CAN conventions.

## A.3 Command Frames (Host → Controller)

| Offset | Command | DLC | Payload |
| --- | --- | --- | --- |
| 0x00 | PING | 0 | (empty) |
| 0x01 | CMD_VEL | 4 | v:int16 w:int16 (×1000) |
| 0x02 | WHEEL | 4 | l:int16 r:int16 (×1000) |
| 0x03 | STOP | 0 | (empty) |
| 0x04 | PRESET | 1 | mode:uint8 (0=restricted, 1=full) |
| 0x05 | ENABLE | 1 | state:uint8 (0=OFF, 1=ON) |
| 0x06 | HB | 0 | (empty) |
| 0x07 | STREAM | 2 | state:uint8 rate:uint8 |
| 0x08 | STATUS_REQ | 0 | (empty) |
| 0x09 | GET | 1 | param_id:uint8 |
| 0x0A | SET | 5 | param_id:uint8 value:int32 (×1000) |
| 0x0B | SAVE | 0 | (empty) |
| 0x0C | LOAD | 0 | (empty) |
| 0x0D | DEFAULTS | 0 | (empty) |

## A.4 Response Frames (Controller → Host)

| Offset | Response | DLC | Payload |
| --- | --- | --- | --- |
| 0x10 | PONG | 0 | (empty) |
| 0x11 | ACK | 2 | cmd_offset:uint8 error:uint8 (0=OK) |
| 0x12 | STATUS_1 | 5 | preset:u8 mode:u8 en:u8 fault:u8 estop:u8 |
| 0x13 | STATUS_2 | 8 | tl:i16 tr:i16 vl:i16 vr:i16 (×1000) |
| 0x14 | STATUS_3 | 7 | dl:i16 dr:i16 vbat:u16 batt:u8 |
| 0x15 | STREAM_1 | 8 | tl:i16 tr:i16 vl:i16 vr:i16 (×1000) |
| 0x16 | STREAM_2 | 6 | dl:i16 dr:i16 vbat:u16 |
| 0x17 | PARAM_VAL | 5 | param_id:u8 value:i32 (×1000) |
| 0x18 | WARN | 3 | type:u8 value:u16 |
| 0x19 | FAULT | 1 | fault_code:u8 |

## A.5 Parameter IDs

Each configuration parameter (Section 7) is assigned a numeric ID for CAN encoding:

| ID | Parameter | ID | Parameter |
| --- | --- | --- | --- |
| 0x00 | kp | 0x08 | slow_timeout |
| 0x01 | ki | 0x09 | normal_timeout |
| 0x02 | kd | 0x0A | hb_timeout |
| 0x03 | counts_per_rev | 0x0B | slow_duty_limit |
| 0x04 | wheel_diameter | 0x0C | normal_duty_limit |
| 0x05 | track_width | 0x0D | batt_full |
| 0x06 | max_accel | 0x0E | batt_low |
| 0x07 | node_id (CAN only) | 0x0F | batt_critical |

IDs 0x10–0x7F are reserved for future standard parameters. IDs 0x80–0xFF are available for vendor-specific parameters.

# Annex B: Extending ORCP (Informative)

## B.1 Vendor-Specific Commands

Implementations MAY add commands beyond those defined in this specification. Vendor-specific commands SHOULD use a prefix to avoid collisions with future ORCP standard commands (e.g. VENDOR_DIAG, X_CALIBRATE). Vendor-specific commands MUST NOT redefine the semantics of standard commands.

## B.2 Additional Telemetry Fields

Implementations MAY add fields to STATUS, STREAM, and other responses. Examples include IMU data (imu_ax, imu_gy, imu_mz), per-motor current (il, ir), temperature (temp), or GPS (lat, lon). Clients MUST ignore unrecognised fields.

## B.3 Additional Kinematic Models

Platforms with non-differential kinematics (Ackermann, omnidirectional, legged) MAY define new commands for their specific needs. RECOMMENDED approach: define a new command (e.g. OMNI_VEL vx= vy= w= for omnidirectional) rather than overloading CMD_VEL. The safety system, configuration, and telemetry commands remain unchanged.

## B.4 Multi-Robot Coordination

For CAN-based multi-robot systems, a broadcast mechanism is RECOMMENDED. A reserved base ID (e.g. 0x000) MAY be used for commands addressed to all devices. The STOP command on the broadcast ID provides a bus-wide emergency stop capability.

# Revision History

| Version | Date | Description |
| --- | --- | --- |
| 1.0 | March 2026 | Initial release. ASCII message format, 15 standard commands, safety system with presets, streaming telemetry, configuration management, CAN binary encoding annex, three conformance levels. |

# License

Copyright © 2026 The ORCP Authors

Permission is hereby granted, free of charge, to any person obtaining a copy of this specification, to use, copy, modify, merge, publish, distribute, sublicense, and/or sell implementations of the protocol described herein, subject to the following conditions:

The above copyright notice and this permission notice shall be included in all copies or substantial portions of the specification.

THE SPECIFICATION IS PROVIDED "AS IS", WITHOUT WARRANTY OF ANY KIND, EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO THE WARRANTIES OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE AND NONINFRINGEMENT. IN NO EVENT SHALL THE AUTHORS OR COPYRIGHT HOLDERS BE LIABLE FOR ANY CLAIM, DAMAGES OR OTHER LIABILITY, WHETHER IN AN ACTION OF CONTRACT, TORT OR OTHERWISE, ARISING FROM, OUT OF OR IN CONNECTION WITH THE SPECIFICATION OR THE USE OR OTHER DEALINGS IN THE SPECIFICATION.

End of specification.